# Safety Response for OARS

#RIVeR

#Ibex

Questions

- OARS wants to know max volt/amp of any circuits being worked on for the vehicles  Max Voltage: 48V

Max Current: 44.856A

Please see detailed specifications of each component listed below.

- Do any components of the electrical systems have "stored energy" - I take this to mean significant capacitors that would hold a charge even if the battery was disconnected .

No component has large amounts of stored energy, and remaining capacitors store negligible energy.

- Is there a list of approved drivers, covered by insurance?  Approved Drivers:

James Tukpah  Katiso Mabulu  Tom Lutz

# Component List

Battery

Kairos System

Hyper Spectral System

Allied Vision Alvium RGB Camera  Short Wave Infrared HSI

Ouster OS1-128

Visible Near Infrared HSI  VNIR Spectrometer

NIR Spectrometer

OnBoard Computer  Thermal Fans

Router

NUC Buck Converter  SWIR Buck Converter

Hyper Spectral Router

# Battery

Model → Lossigy LiFePO4 Lithium Battery  Voltage → 48V

Current → 100A

Power → 5120Wh

# Kairos Autonomy Sub-System

## Kairos System

Model → Pronto4 Series 4 (P4S4)  Voltage → 12V

Current → 5A

5A when not driving, or running actuators  20A running Actuators without engine

10A running actuators and engine  Power → 240W

# Hyper Spectral Camera Sub-System

## Allied Vision Alvium RGB Camera

Model → Alvium 1800 U-507  Voltage → 5V

Current → 0.4A  Power → 2.0 W

## Short Wave Infrared (SWIR) HSI

Model → Imec SNAPSHOT SWIR 16 Bands  Voltage → 6V

Current → 3A

Power → 18W

## Visible Near Infrared (VNIR) HSI

Model → Ximea Snapshot VIS

Voltage → 5V

Current → 0.36A  Power → 1.8W

## Ouster OS1-128 Lidar

Model: OS1-128  Voltage → 24V

Current → 0.66667A

Power → 16W (Nominal)

## VNIR Spectrometer

Model → Pebble VNIR  Voltage → 6V

Current → 0.1333A

Power → 0.8W

## NIR Spectrometer

Model → Pebble NIR  Voltage → 6V

Current → 0.1333A  Power → 0.8W

## NUC Computer

Model → NUC11PHBi7  Voltage → 19.5V

Current → 11.7A  Power → 230 W

## Thermal Fan (Front Fan)

Model → Rittal 3237.124  Voltage → 24V

Current → 0.1A  Power → 2.4W

## Thermal Fan (Back Fan)

Model → Rittal 3237.124  Voltage → 24V

Current → 0.125A

Power → 3 W

## NUC Buck Convertor

Model → Noyito 20 A Buck Converter  Voltage → 24V

Input Voltage range → 6V-40V

Output Voltage Range → 1.25V-36V  Current → 20A

Output Current range → 0.2A-20A

Max Output Current → 20A  Power → 300W

## SWIR Buck Convertor

Model → Drok DC Buck Converter  Voltage →

Input Voltage range → 5.3V-32V

Output Voltage Range → 1.2V-32V  Current → 8A

Output Current → 8A

Max Output Current → 12A

Working current → 25mA = 0.025A  Power → 120W

# Hyper Drive Router Sub-System

## Router

Model → TP-Link Archer A7 V5 Voltage → 12V

Current → 1.5A Power → 18 W